# NON-DESTRUCTIVE EVALUATION OF MATERIALS (20MEP02)

### Unit 3 – Thermography & Eddy Current Testing | 30 MCQ Questions with Answers

1. What is Thermography in the context of NDT?

- A) A method that uses sound waves to detect defects

- B) An NDT method that detects defects by measuring temperature variations on the surface of a component

- C) A technique that uses magnetic fields to identify cracks

- D) A method that uses X-rays to image internal defects

Answer: B) An NDT method that detects defects by measuring temperature variations on the surface of a component

2. Which of the following best describes Active Thermography?

- A) The component is inspected using its natural heat emission without any external energy input

- B) An external energy source is applied to the component to induce thermal contrast for defect detection

- C) The component is cooled below ambient temperature for inspection

- D) Infrared cameras are used without any stimulation of the component

Answer: B) An external energy source is applied to the component to induce thermal contrast for defect detection

3. Which of the following best describes Passive Thermography?

- A) External heat is applied to the component before inspection

- B) The component's natural heat emission during operation is monitored to detect anomalies

- C) The component is submerged in liquid for thermal imaging

- D) UV light is used to stimulate thermal emission from the component

Answer: B) The component's natural heat emission during operation is monitored to detect anomalies

4. Liquid Crystal Thermography works on the principle that:

- A) Liquid crystals emit infrared radiation proportional to defect depth

- B) Liquid crystals change colour in response to temperature variations, revealing thermal patterns

- C) Liquid crystals conduct electricity differently over defects

- D) Liquid crystals fluoresce under ultraviolet light near crack regions

Answer: B) Liquid crystals change colour in response to temperature variations, revealing thermal patterns

5. Which of the following is an advantage of Liquid Crystal Thermography?

- A) It can detect very deep subsurface defects up to several centimetres

- B) It provides a full-field, real-time colour map of surface temperature distribution

- C) It does not require any surface preparation

- D) It works on all metallic and non-metallic materials without calibration

Answer: B) It provides a full-field, real-time colour map of surface temperature distribution

6. Which of the following is a limitation of Liquid Crystal Thermography?

- A) It cannot measure temperature at all

- B) It has a narrow operating temperature range and requires direct contact with the surface

- C) It requires radioactive isotopes for activation

- D) It can only be used on ferromagnetic materials

Answer: B) It has a narrow operating temperature range and requires direct contact with the surface

7. Infrared Thermography detects defects by measuring:

- A) The electrical resistance across the component surface

- B) Infrared radiation emitted from the component surface, which varies with subsurface defects

- C) The magnetic field strength at the component surface

- D) The acoustic emission from propagating cracks

Answer: B) Infrared radiation emitted from the component surface, which varies with subsurface defects

8. The electromagnetic spectrum region used in Infrared Thermography is:

- A) Ultraviolet region

- B) Infrared region (wavelengths between 0.75 µm and 1000 µm)

- C) X-ray region

- D) Visible light region only

Answer: B) Infrared region (wavelengths between 0.75 µm and 1000 µm)

9. Which of the following is a type of IR detector used in Infrared Thermography?

- A) Piezoelectric transducer

- B) Photon detector and thermal detector

- C) Geiger-Müller tube

- D) Scintillation counter

Answer: B) Photon detector and thermal detector

10. In Infrared Thermography, a defect appears as a ______ spot on the thermal image compared to the surrounding area.

- A) Same temperature

- B) Hot or cold spot depending on the nature and location of the defect

- C) Always a cold spot

- D) Always a hot spot

Answer: B) Hot or cold spot depending on the nature and location of the defect

11. Which of the following is an application of Infrared Thermography in NDT?

- A) Detecting surface cracks in aluminium using penetrant liquids

- B) Detecting delaminations in composites, heat loss in buildings, and electrical hotspots in circuits

- C) Measuring the hardness of metallic surfaces

- D) Identifying grain boundaries in metals using magnetic fields

Answer: B) Detecting delaminations in composites, heat loss in buildings, and electrical hotspots in circuits

12. Which of the following is an inspection method used in Active Thermography?

- A) Passive emission monitoring

- B) Pulsed thermography, lock-in thermography, and vibrothermography

- C) Continuous wave ultrasonic stimulation only

- D) Magnetic pulse heating

Answer: B) Pulsed thermography, lock-in thermography, and vibrothermography

13. What instrumentation is primarily used to capture thermal images in Infrared Thermography?

- A) Ultrasonic flaw detector

- B) Infrared camera (thermographic camera)

- C) Magnetic yoke with Hall effect sensor

- D) X-ray film and developer

Answer: B) Infrared camera (thermographic camera)

14. Which of the following is an advantage of Infrared Thermography as an NDT method?

- A) It requires physical contact with the component for accurate results

- B) It is a non-contact, full-field inspection method capable of scanning large areas quickly

- C) It can detect only metallic defects

- D) It requires the component to be ferromagnetic

Answer: B) It is a non-contact, full-field inspection method capable of scanning large areas quickly

15. In thermography, emissivity refers to:

- A) The ability of a material to conduct electricity

- B) The efficiency of a surface in emitting infrared radiation compared to a perfect blackbody

- C) The temperature gradient across a defect

- D) The reflectivity of a surface to ultrasonic waves

Answer: B) The efficiency of a surface in emitting infrared radiation compared to a perfect blackbody

16. What is the basic principle of Eddy Current Testing (ECT)?

- A) A penetrant liquid is drawn into surface defects by capillary action

- B) A changing magnetic field induces eddy currents in a conductive material, and defects alter these currents, which is detected by the probe

- C) Ultrasonic pulses are reflected from internal discontinuities

- D) Magnetic particles are attracted to flux leakage points at defects

Answer: B) A changing magnetic field induces eddy currents in a conductive material, and defects alter these currents, which is detected by the probe

17. Eddy currents are induced in a conductive material when:

- A) A static magnetic field is applied to the surface

- B) An alternating magnetic field from a coil is brought near the electrically conductive material

- C) Ultrasonic waves pass through the material

- D) The material is heated above its Curie temperature

Answer: B) An alternating magnetic field from a coil is brought near the electrically conductive material

18. Which of the following materials can be tested using Eddy Current Testing?

- A) Non-conductive materials like wood and ceramics

- B) Electrically conductive materials such as metals and alloys

- C) Only ferromagnetic materials

- D) Only non-ferrous metals like aluminium and copper

Answer: B) Electrically conductive materials such as metals and alloys

19. In Eddy Current Testing, the presence of a defect is detected by observing:

- A) A change in the colour of the component surface

- B) A change in the impedance of the test coil caused by the altered eddy current flow

- C) A temperature rise at the defect location

- D) Reflection of magnetic particles at the crack boundary

Answer: B) A change in the impedance of the test coil caused by the altered eddy current flow

20. The depth to which eddy currents penetrate into a material is called:

- A) Wavelength

- B) Standard depth of penetration or skin depth

- C) Attenuation depth

- D) Diffraction depth

Answer: B) Standard depth of penetration or skin depth

21. How does the test frequency affect the depth of penetration in Eddy Current Testing?

- A) Higher frequency increases the depth of penetration

- B) Lower frequency increases the depth of penetration into the material

- C) Frequency has no effect on penetration depth

- D) Higher frequency is always used for subsurface defect detection

Answer: B) Lower frequency increases the depth of penetration into the material

22. Which of the following is an element (component) of an Eddy Current Testing system?

- A) Penetrant applicator and developer spray

- B) Test coil (probe), AC power source, impedance measuring instrument, and display unit

- C) Magnetic yoke and fluorescent particles

- D) X-ray tube, film, and darkroom processing equipment

Answer: B) Test coil (probe), AC power source, impedance measuring instrument, and display unit

23. Which type of Eddy Current probe is commonly used for detecting surface cracks in flat components?

- A) Encircling coil

- B) Surface (pancake) probe

- C) Internal bobbin coil

- D) Differential through-transmission coil

Answer: B) Surface (pancake) probe

24. An encircling coil in Eddy Current Testing is used for:

- A) Inspecting flat plate surfaces for corrosion

- B) Inspecting cylindrical components like tubes, rods, and bars as they pass through the coil

- C) Detecting deep subsurface defects in thick sections

- D) Measuring the surface roughness of components

Answer: B) Inspecting cylindrical components like tubes, rods, and bars as they pass through the coil

25. Which of the following properties of a material can be measured using Eddy Current Testing?

- A) Tensile strength and hardness only

- B) Electrical conductivity, magnetic permeability, coating thickness, and detection of cracks

- C) Grain size and fracture toughness

- D) Melting point and thermal conductivity only

Answer: B) Electrical conductivity, magnetic permeability, coating thickness, and detection of cracks

26. Which of the following is an application of Eddy Current Testing?

- A) Detecting internal porosity in castings using sound waves

- B) Inspection of heat exchanger tubes, aircraft fuselage skin, and detection of cracks in conductive components

- C) Measuring wall thickness of non-conductive polymer pipes

- D) Detecting surface cracks in ceramics using penetrant liquids

Answer: B) Inspection of heat exchanger tubes, aircraft fuselage skin, and detection of cracks in conductive components

27. Which of the following is a limitation of Eddy Current Testing?

- A) It cannot detect surface cracks

- B) It is limited to electrically conductive materials and has limited depth of penetration

- C) It requires the component to be submerged in water during inspection

- D) It can only be performed at elevated temperatures

Answer: B) It is limited to electrically conductive materials and has limited depth of penetration

28. In Eddy Current Testing, the 'lift-off effect' refers to:

- A) The increase in eddy current penetration as the probe is lifted from the surface

- B) The change in probe impedance caused by variations in the distance between the probe and the test surface

- C) The reduction in magnetic field strength due to component geometry

- D) The signal noise generated by surface roughness of the component

Answer: B) The change in probe impedance caused by variations in the distance between the probe and the test surface

29. Which of the following is a key advantage of Eddy Current Testing over Magnetic Particle Testing?

- A) ECT can test ferromagnetic materials only

- B) ECT can be applied to all electrically conductive materials including non-ferromagnetic metals, and requires no contact or surface preparation

- C) ECT always provides greater depth of penetration than MPT

- D) ECT does not require any instrumentation or calibration

Answer: B) ECT can be applied to all electrically conductive materials including non-ferromagnetic metals, and requires no contact or surface preparation

30. A case study application of Eddy Current Testing in the aviation industry involves:

- A) Using penetrant liquids to detect cracks in composite panels

- B) Detecting fatigue cracks around fastener holes in aluminium aircraft structures during maintenance inspections

- C) Using gamma rays to image internal defects in engine components

- D) Applying magnetic particles to detect cracks in titanium landing gear components

Answer: B) Detecting fatigue cracks around fastener holes in aluminium aircraft structures during maintenance inspections